# Sensores from Home assistant:

PV inverter :

sensor.deye8k_pv_power

sensor.deye8k_battery (SOC)

sensor.deye8k_battery_power

sensor.deye8k_grid_power

sensor.deye8k_load_power

number.deye8k_battery_charging_power

number.deye8k_battery_discharging_power

switch.deye8k_battery_grid_charging

electricity price (dynamical)

sensor.octopus_a_10fc0646_electricity_price

Wetter forecast:

weather.openweathermap

PV-forecast:

sensor.solcast_pv_forecast_prognose_heute

sensor.solcast_pv_forecast_prognose_morgen

sensor.solcast_pv_forecast_prognose_verbleibende_leistung_heute

# additional config fields :

prices:

Cheap Rate Threshold (€/kWh) default=0,28

feed-in tariff for solar power (€/kWh) default = 0,08

fix_price (€) default 0,30

inverter:
Max Charge Power (W)  default=5500
Max Discharge Power (W) default=5500

Battery:

Capacity (kWh) default: 25

min SOC Default 10

max SOC Default 95

# internal calculated Sensores published to HA as mqtt device / Sensors:

Battery kwh freetochange =  max SOC – SOC/100 * Battery Capacity

Battery kwh useable = (SOC – min SOC) / 100 * Battery Capacity

predicted_load_kwh = s. Logik 2) prediction

real_costs (€)

cost_without_solar (€)

cost_without_grid_charge (€)

cost_fix_price_tarif (€)

# Logic

1) Logic   Grid charge→Battery

simulation mode (log only)

Battery Control mode (controll inverter to charge battery from grid)

schedule  every 5 min:

if energy price < „Cheap Rate Threshold“

and Battery kwh freetochange > sensor.solcast_pv_forecast_prognose_verbleibende_leistung_heute

→ Grid Charge = on and number.deye8k_battery_discharging_power =0 (charge from grid and do not use battery to load)

if not  Grid Charge = off and number.deye8k_battery_discharging_power =185

2) Prediction

save todays power consumption, todays high&low temperature in local DB

predicted_load_kwh= get temperatur high & low from weather.openweathermap

search for similar temperatur vaulues in DB → get power consumption of simular day

if no simular days in DB → show in FrontEnd (estimate)

then if min temp <0 and max temp < 0 = 30 kWh

then if min temp <0 and max temp < 10 = 20 kWh

then if min temp > 0 and max temp < 15 =10 kWh

3) Track energy prices and consumption

alle prices as today, monthly, yearly, total (save in Addon DB) → publish to HA

track power deltas to calculate costs, reboot and error resiliens (Check whether the value is realistic (last value vs. current value; if unrealistic, use the next value for the delta calculation) save last value for resilience)

cost_without_solar (€) = (load_power * price)

cost_fix_price_tarif (€) =  (load_power * fix_price)

additional changes:

remove APP configuration settings, configuration is only done in the FrontEnd
Show info in Frontend, if nesessary configuration is missed

Show info in Frontend, if nesessary sensors have no value

Show Log in Fronteend (grid charge mode changed with on/off, dateTime, Battery kwh freetochange, Battery kwh useable, predicted_load_kwh)